This workbook outlines the essential steps and validation procedures for the Security Setup phase of the DFSP Onboarding Playbook, drawing upon the technical implementation plan for a Hub Deployment. The security setup represents a critical and often time-consuming phase of onboarding [User Query].

The steps below detail the configuration of security elements necessary for a Participant System (DFSP Deployment) to securely connect and authenticate with the Hub Cluster.

Security Configuration Workbook: DFSP Onboarding

I. Network Allowlisting (VPN Configuration and IP Allowlist Setup)

This section details the procedures for establishing secure network access, including setting up WireGuard VPN access and configuring Egress Endpoints in the Mojaloop Connection Manager (MCM).

A. VPN Configuration (Secure Network Access)

Establishing a WireGuard VPN connection is crucial for secure network access to the DFSP/Hub Cluster.

B. IP Allowlist Configuration in Hub (MCM Setup)

The DFSP must provide its external-facing IP addresses (NAT Gateway IP) for configuration as Egress Endpoints in the Mojaloop Connection Manager (MCM).

| Step | Description | Source Reference | Validation Script / Action |
| --- | --- | --- | --- |
| 1. Identify DFSP Endpoints | Get the NAT gateway IP address of the DFSP Cluster, which represents the DFSP's endpoints (e.g., Wallet 1 and Wallet 2). This IP address must be provided to the Hub Operator. |  | Action: Retrieve NAT Gateway IP address of the DFSP Cluster. |
| 2. Access MCM UI | The Hub Operator retrieves MCM secret credentials from the Hub Cluster’s Vault. The Operator then accesses the MCM ingress URL, located in the Hub Kubernetes Cluster’s “mcm” namespace. |  | Action: Log in to the MCM UI. |
| 3. Configure Egress Endpoints (DFSP Action) | The DFSP enters its IP addresses in the MCM UI under Endpoint Configuration > Egress Endpoints for each wallet. |  | Configuration Example: Egress address: NAT gateway ip of DFSP [ ip x.x.x.x/32 Port: 443]. Action: Click the “Submit for Confirmation” button after adding IPs. |
| 4. Approve Endpoints (Hub Operator Action) | The Hub Operator navigates to Hub -> Unprocessed Endpoints, locates the IPs for the DFSP, and confirms them by clicking the “Confirm Selected Endpoints” button. |  | Action: Confirm Endpoints in MCM UI. |

II. Certificate Exchange (Mutual TLS Authentication)

The MCM manages security using TLS certificate configuration and facilitates the certificate exchange required for Mutual TLS (mTLS) authentication with the switch.

| Step | Description | Source Reference | Validation Script / Action |
| --- | --- | --- | --- |
| 1. Sign DFSP CSR | The Hub Operator navigates to Hub -> TLS Client Certificates -> Unprocessed DFSP CSRs. The operator locates the Certificate Signing Request (CSR) for the DFSP and clicks the “Use Provided CA To Sign CSR” button. |  | Action: Hub signs the DFSP CSR in the MCM UI. |
| 2. Onboard DFSP | The Hub Operator proceeds to Hub -> DFSPs, and clicks the “onboard” button (marked with an upload icon) next to the DFSP name. |  | Validation: Wait 60 seconds for all configurations to be propagated. |

III. JWS Key Configuration (Message Integrity)

JSON Web Signature (JWS) keys are used for message integrity.

| Step | Description | Source Reference | Validation Script / Action |
| --- | --- | --- | --- |
| 1. JWS Key Exchange | For onboarding the DFSP to the Hub, certificates and JWS keys are exchanged between the Hub and the DFSP Deployment. |  | Action: This exchange is initiated by the Hub Operator as part of the DFSP onboarding process, which includes the certificate exchange (Step II, above). |
| 2. Test Signature Generation and Validation | (Specific step-by-step validation scripts for JWS signature generation and validation across all endpoints are not detailed in the provided sources, but successful onboarding confirms the exchange occurred.) | N/A | Validation Insight: Successful execution of initial transaction testing and API conformance testing will validate the integrity provided by the JWS configuration. |

I. OAuth Token Setup (API Authentication and Access Control)

This configuration ensures proper API authentication flows, access control, and platform stability before DFSP onboarding.

| Step | Description | Source Reference | Validation Script / Action |
| --- | --- | --- | --- |
| 1. Generate DFSP secret | Create a DFSP client secret in the Hub’s Keycloak and share it securely with PM4ML for OAuth authentication to Hub. PM4ML need to add secret in the Vault. |  | Validation Insight: Successful login and API calls using OAuth tokens in MCM Client/Management API in PM4ML. |